DCR Road and Trail Metadata

David Kimball, DCR GIS

david.kimball@state.ma.us

617-626-1447

May 2006

This document describes the fields (columns) in the DCR Road and Trail shapefiles that are created/edited with GPS fieldwork.  This document is intended to be used both as metadata (to describe the data for the end user) and as a reference for the person collecting data in the field using GPS.  This metadata is also for use with the DCR Road and Trail Data Dictionary, which works with non-ArcPad Trimble GPS units.

Lines (DCR_trail_road.shp)

This shapefile has attributes that describe road and trail sections.  A section is a part of a road or trail that runs between two intersection points.  If a section of trail changes significantly (for instance if it widens from 0’-3’ to 5’-10’, or it changes from gravel to dirt, or the condition deteriorates significantly) the field user should finish the line feature, enter the attributes for the first section, then start a new line feature so that the second section will have the correct attributes.  Minor changes or variations in the trail or road can be ignored (give the entire line the average or minimum attributes for the trail or road).

Certain linear features, if less than 50 feet long, can be collected as points.  A short boardwalk, for instance, can be collected as a Trail Stream Crossing point.  A longer boardwalk would be collected as a line (with Surface of Wood or Plastic).  A short section of damaged trail or road could be collected as a Trail Damage or Road Damage point, while a long section would be its own separate line with Condition of Poor or Fair (in this instance you should also take a Trail Damage or Road Damage point somewhere along the damaged section so you can enter the additional attributes that go with that point type).

Type: Trail, Forest Road/Trail, Administrative Road, Public Road, Other; what type of road or trail is this section?

Surface: Natural, Asphalt, Concrete, Processed Gravel, Bank Run Gravel, Vegetated, Stone and Oil, Packed Stone Dust, Pavers, Wood, Plastic, Other; the predominant surface of the road or trail section.  “Plastic” would be used for recycled plastic boardwalks, and “Wood” would be used for wooden boardwalks, etc.

Width: 0' - 3', 3' - 5', 5' - 10', 10' - 15', 15' - 20', 20' +; minimum width of the section of trail or road.  Slight variations are acceptable but if the trail changes width significantly at some point you should stop the line there and start a new one so they both have accurate width attributes.

Condition: Good, Fair, Poor; the condition of the road/trail.  This is to flag road and trails for maintenance.  A hiking trail can be quite rugged yet still be in good condition if it does not seem to require maintenance.  So the condition indicates the road/trail’s fitness for its intended use.

Vehicacces: Car, 2WD Truck, 4WD Truck, ORV, None; what type of vehicle can access this trail or road.  The most accessible roads can be accessed by Car, and the least accessible trails can not be accessed by any vehicle (“None”).  So a road coded “4WD Truck” can be accessed by 4WD Truck, ORV, and on foot, but cannot be accessed by a Car or 2WD Truck.

Trail_mark: None, Good, Fair, Poor; the quality/existence of trail marking (blazes or signs indicating the course of the trail, so that the user knows they have not wandered off the trail.  These are typically paint marks on trees or rocks, or plastic/metal colored shapes nailed to trees (for instance a blue triangle or a small sign with a picture of a paw print)).  This is for trails only.  None = no trail markers; Good = clear trail markers all along trail; Fair = trail markers along only part of trail or some markers hard to see; Poor = trail markers are misleading or in need of re-blazing.

Illegal: (True/False); if the user thinks the trail (or road?) might be illegal/unofficial, they can indicate this here.  Default is F (False).  Marking a trail as “True” flags the trail for review by the park supervisor, who will decide whether the trail should be closed off.

Comments: (text); the user can optionally enter a comment if there is anything unusual about the line or anything that needs to be explained more clearly than is possible with the standard attributes.

Source: (text); this is entered back in the office, not in the field.  It indicates the source of the data, including whether the line was GPSed or digitized, and whether it was differentially corrected or not.

GPS_Date: (text); holds the date the feature was GPSed in the field.  Autogenerated by the GPS unit.

GPS_Time: (text); holds the time the feature was GPSed in the field.  Autogenerated by the GPS unit.

Points (DCR_trail_road_points.shp)

This shapefile can hold a large number of different types of points related to assets and features along the roads and trails.  During fieldwork, many types of features shall be collected as averaged, multi-position GPS points.  The most important attribute field is Type.  Most of the rest of the fields will be blank; only a few will be filled in, depending on the Type of point (most of the fields are associated exclusively with one type).  Many of the feature types are specific to either Roads or Trails, so feature types whose names start with “Road” should only exist on or near a Road; likewise feature types whose names start with “Trail” should only exist on or near a Trail.  Be careful as there are some point features that are different Types depending on whether you are on a road or a trail (culverts, bridges, damage, signs, etc.).  The features that do not start with “Road” or “Trail” can be placed anywhere.

The fields (for the most part) are named according to a two-part scheme.  Since the .dbf format allows only 10-character field names, the first 5 characters indicate the Type the field is associated with, and the last 5 characters indicate the specific attribute.  Some fields are not Type-specific.  Attributes are sorted below by Type.  Field names are underlined.

Type:  The type of point.  This is the first thing the user chooses, and then they fill out the fields that correspond to that Type.  Types are listed below (in bold), followed by a list of their corresponding attribute fields (underlined) and acceptable values (separated by commas) and a brief description (in italics, after the ; semicolon) of the attribute field.  Some acceptable values are in quotes for clarity (usually because they contain a comma).  Attribute fields that are not picklists have their data type (text, integer, number, True/False checkbox) in parentheses in place of the list of acceptable values.  At the end are listed a few fields that are available for all Types.  The types and their attribute fields are described in this format:

NameOfPointType – [optional note about this Type goes here in square brackets]

Fieldname: PossibleValue1, PossibleValue2, PossibleValue3; description of field.

[or]

Fieldname: (InputType*); description of field.

* i.e. text, integer, number, True/False

Intersection – [any place where three or more trails meet]

Num: 1, 2, 3, 4, 5, 6789; this is the number of trails that meet at that intersection, including the one you walked in on.  A real intersection will have a Num of at least 3 (the numbers 1 and 2 should only be used for the Trailhead point Type – see below).  “6789” means “6+”, i.e. 6 or more (the user sees “6+” in the field, but the value written to the attribute table is “6789”).

Intsignage: Good, Fair, Poor; trail name signage at this intersection.  Good = you can tell the names of all trails at this intersection without difficulty from the signage; Fair = you can tell the names of some of the trails (or of all the trails but with difficulty); Poor = signage is misleading or confusing; None = there is no trail name signage at this intersection.

Int_number: (text); if there is a sign on a tree/post/stake with an intersection number, type the number here.  In some parks the intersections are numbered to help people find where they are on the map.

Trailhead – [the access point for a trail/road – usually near a parking lot or public road]

The attributes for trailheads are exactly the same as for intersections.  In this case Num is the number of trails that start at this point.

Dead End

No additional attributes necessary.

Parking Area

Prkngsurf: Paved asphalt, Processed gravel, Bank run gravel, Natural surface, Other; primary surface material of the parking area.

Prkngcond: Good, Fair, Poor; overall condition of the parking area.

Prkngcurbm: None, Granite, Asphalt, Concrete, Other; primary material the parking area’s curbing is made of (or None if there is no curbing).

Prkngdtype:  Catchbasin, Sheet flow, Concentrated outlet, Other; drainage type of the parking lot (a concentrated outlet is a small gap in the curbing).  It may be necessary to take additional point features to represent some of these drainage features.

Prknghandi: (integer); the number of marked handicapped spots in this parking area.

Prkngcapac: (integer); the number of lined boat spaces (extra-long parking spaces that can fit a boat trailer) in this parking area.

Prkngcapac: (integer); the total number of cars intended to fit in this parking area (not necessarily referring to painted-line spaces; includes handicapped and boat spaces if they are present).

Prkngesize: (text); approximate size in feet (Length x Width)

Prkngcurbs: (True/False); does the curbing (if present) only partially surround the parking lot?

Prknglined: (True/False); does the parking lot have painted lines?

Prkngswale: (True/False); is there a swale present (a swale is a small opening in a curb that allows water to drain out of the parking lot)?

Prkngmnt_p: (True/False); does the parking lot require maintenance?

Prkngmnt_c: (True/False); does the curbing (if present) require maintenance?

Gate – [any structure that is used to block vehicular access to the road or trail system—see list of gate types below]

Gate_effec: (True/False); does gate effectively block motorized vehicles?

Gate_owner: DCR, Other state agency, Municipal, Federal, Private, Non-profit, Other; the group or entity that has control over the gate.

Gate_num: (text); the gate number (if gate has a number on it).

Gate_type: Steel channel (HD square pipe), Steel pipe (normal round pipe), Farm gate, Wire or cable, Treated timber, Untreated timber, Log, Gravel pile, Rip rap large rock in pile, Posts, Jersey barrier, Other; the type of gate.

Gate_posts: Wood post, Treated post, Steel post, Well casing or heavy pipe, Other; the type of gate posts.

Gate_rflct: (True/False); is there red reflective tape in good condition on the gate?

Gate_confg: Single lane, Two lanes with no center post, Two lanes with center post; the configuration of the gate.

Gate_cond: Good, Fair, Poor; the condition of the gate.  Good = like new materials and treatment OK; Fair = has rust, deterioration or damage, but is repairable; Poor = failed, not economical to repair.

Gate_heigh: (number); gate height to nearest ½ foot.

Gate_width: (integer); gate width to nearest foot.

Road Bridge – [see diagram in Road manual]

Rbridowner: DCR, Other state agency, Municipal, Federal, Private, Non-profit, Other; the group or entity that has control over the bridge.

Rbridspan: (integer); the actual distance the bridge is suspended (see diagram) to the nearest foot.

Rbriddmate: “Untreated wood, w/running plank”, “Untreated wood, no running plank”, “Treated wood, w/running plank”, “Treated wood, no running plank”, Glulam, Steel plate, Expanded steel, Concrete, Stone, Other; the primary material of which the bridge’s deck is made.

Rbriddwidt: (integer); the width of the bridge’s deck that can be driven on, to the nearest inch.

Rbriddleng: (integer); the length of the top or driving surface of the bridge’s deck, to the nearest foot.

Rbriddcond: Good, Fair Poor; the condition of the bridge’s deck.  Good = [Wood: weathered, no stain, moss, soil, or insects.  Concrete: no spalling or cracks.  Steel: bright or paint, no rust.]; Fair = [Wood: some stain, moss or soil present, minor incipient decay and/or insects present.  Concrete: minor spalling or cracks.  Steel: minor rust, fasteners tight.]; Poor = [All: voids or missing parts.  Wood: advanced decay and/or insects present.  Concrete: major spalling or cracks.  Steel: major rust, fasteners loose.].

Rbridowidt: (integer); width of opening (distance, to nearest foot, of opening between abutments under bridge (see diagram)).

Rbridoheig: (integer); height of opening (distance, to nearest foot, of opening from stream bottom to lowest point on bridge (see diagram)).

Rbridsmate: Steel, “Sawn, treated wood”, “Sawn, untreated wood”, “Roundwood, treated”, “Roundwood, untreated”, Glulam, Concrete, Other; stringer material.

Rbridscond: Good, Fair, Poor; stringer condition.  Good = [Wood: weathered, no stain, moss, soil, or insects.  Concrete: no spalling or cracks.  Steel: bright or paint, no rust.]; Fair = [Wood: some stain, moss or soil present, minor incipient decay and/or insects present.  Concrete: minor spalling or cracks.  Steel: minor rust, fasteners tight.]; Poor = [All: voids or missing, rot in wood members.  Wood: advanced decay and/or insects present.  Concrete: major spalling or cracks.  Steel: major rust, fasteners loose.].

Rbridsnum: (integer); number of stringers.

Rbridssize: (text); stringer dimension (Height” x Width” x Length’) (see diagram).

Rbridhrati: “< H-10”, H-10, H-15, H-20, H-25, Failed, Unknown; “H” Rating (AASHTO-44 Rating): select form from list, if there is no documentation from an authoritative source, enter “Unknown”.

Rbridamate: Quarried stone, Field stone, Rubble, Concrete, Treated wood cribbing, Untreated wood cribbing, Other/combination; primary abutment material.

Rbridacond: Good, Fair, Poor; abutment condition.  Good = no slumping; Fair = slumping and/or undercutting, load-bearing impaired; Poor = collapsed.

Rbridyeari: (4-digit integer year); year the bridge was installed.

Rbridfcros: (text); name of the feature crossed (Mill River, Elm Street, etc.), or if name is not determinable, type of feature crossed (stream, brook, road, etc.).

Road Culvert – [see diagram in Road manual]

Rculvowner: DCR, Other state agency, Municipal, Federal, Private, Non-profit, Other; the group or entity that has control over the culvert.

Rculvfcros: (text); name of the water feature crossed (Mill River, Muddy Brook, etc.), or if name is not determinable, type of water feature crossed (stream, brook, etc.).  If the culvert is a road drainage structure, enter the road name.

Rculvtype: “Corrugated, galv. steel”, “Corrugated, aluminum”, Iron pipe, Concrete pipe, Corrugated PVC or ABS plastic, “Smooth Hi-Q, PVC or ABS plastic”, Smooth-walled box, Concrete box, Multiple installation, Stone, Other; the type of material that best describes the culvert.

Rculvlengt: (integer); length of culvert, to nearest foot from end to end.

Rculvdiame: (integer); FOR ROUND CULVERTS ONLY diameter to nearest inch.

Rculvheigh: (integer); FOR BOX CULVERTS ONLY height to nearest inch.

Rculvwidth: (integer); FOR BOX CULVERTS ONLY width to nearest inch.

Rculvcover: Adequate, Inadequate; adequacy of cover over pipe; varies by culvert type and size.

Rculvflowc: (text); percent of original flow that the culvert currently allows, rounded to nearest 10%.

Rculvccond: Good, Fair, Poor; condition of culvert structure.  Good = perfect condition; Fair = surface rust and/or less than 20% crushed; Poor = rusted through and/or more than 20% crushed.

Rculvicond: Good, Fair, Poor; condition of culvert inlet.  Good = perfect condition; Fair =  less than 50% blockage; Poor = more than 50% blockage.

Rculvocond: Good, Fair, Poor; condition of culvert outlet.  Good = perfect condition; Fair =  less than 50% blockage; Poor = more than 50% blockage.

Rculvyeari: (4-digit integer year); year the culvert was installed.

Road Drainage Dip – [see diagrams in Road manual]

Rddiptype: Broad based dip, Rolling dip; generally speaking there are two kinds: rolling and broad-based.  Rolling Dips are generally shorter and less sophisticated than Broad Based Dips.  Select the type (from the diagrams) which the dip you are looking at most closely resembles.

Rddipcond: Good, Fair, Poor; condition of the drainage dip.  Good = perfect condition, functions well; Fair = losing definition or washing out, needs re-grading, may have blockage from in/out flow; Poor = making road worse, totally blocked, lost all function.

Road Ford

Rfordfcros: (text); name of the water feature crossed (Mill River, Muddy Brook, etc.), or if name is not determinable, type of water feature crossed (stream, brook, wetland, etc.).

Rfordcond: Good, Fair, Poor; overall condition of ford.  Good = use has not had adverse effect on water quality, banks solid; Fair = useable, may damage water quality under certain conditions, banks may fail; Poor = not useable, is currently damaging water quality, banks currently eroding.

Rfordbtype: Ledge, Stony, Mud, Gravel, Concrete pad, Log or timber, Other; bottom type of the ford.

Road Damage – [if damage is longer than 50 feet, there should also be a separate line segment (with attributes showing condition of road) to show the extent of the damage (the road line leading up to the damage should stop at the beginning of the damage, then a new line for the duration of the damage, then a new line starting after the damage).  Regardless of the length of damage, there should be a Road Damage point– so for a long damaged area, you will have both a line and a point]

Rdamgtype: Washout, Standing water/mud hole, Large boulders, Ledge, Ruts, Multiple, Other; the type of road damage.

Rdamgesize: (text); estimated size of damage (Length’ x Width’ x Depth”).

Rdamgmainr: Failed bridge, Failed culvert, Failed or nonexistent ditch, Frost heave, Lack of crown, Lack of drainage, Not applicable, Other; maintenance related – if possible to identify what caused damage, select from this list.

Rdamguse_r: Not maintained, Vandalism/unauthorized use, Lack of access control, Seasonal damage, Beaver, Not applicable, Other; use related – if possible to identify what caused damage, select from this list.

Rdamgvpass: Car, 2WD Truck, 4WD Truck, ORV, None; what type of vehicle can get past this road damage.  See the Vehicacces attribute of the Trail/Road Line feature type for more detail.

Road Landing Entrance – [all established landings, regardless of level of maintenance or activity]

Rlandesize: (text); estimated size of the landing (Length x Width) in feet.

Rlandmaint: Yes, No; is the landing maintained in some fashion?  Yes = The landing is maintained at some level; No = The landing is being allowed to return to a natural state.

Rlandcover: Bare soil, Grass/herbaceous, Shrub/sapling, Sapling/pole, Other; the ground cover of the landing.

Rlandlastu: (4-digit integer year); the year of the last time this landing has been used for a harvest operation.

Road Sign – [a sign associated with a Road; NOT a trail intersection number sign, trail marker, boundary marker, trail sign, or trail name sign – these are all taken care of by other feature types or their attributes]

Rsignttext: (text); the text of the sign, or what type of sign it is if it is impossible to type in the sign text.

Rsignmater: Steel painted, Aluminum, Plywood, Treated timber, Wood painted, Wood routed, Fiberglass, Other; the primary material of which the sign is made.

Rsignpostm: Treated timber, Steel post, Painted or untreated wood, Log post, Other metal post, Fiberglass post, Tree, Other; the primary material of which the sign post(s) is/are made.

Rsigncond: Good, Fair, Poor; the condition of the sign.  Good = sign and post are properly installed and visible; Fair = sign face or post is in less than desirable condition or location could be improved but the sign is still readable; Poor = either the sign or post is in need of replacement or the location is poor and should be relocated.

Road Turn Out – [see diagram in Road manual]

Rtrnolengh: (integer); length to nearest foot from end to end of tapers.

Rtrnowidth: (integer); width to nearest foot from shoulder to outside edge.

Rtrnosurf: Paved asphalt, Processed gravel, Bank run gravel, Natural surface, Other; the primary surface material of the turn out.

Road Turn Around – [see diagram in Road manual]

Rtrnalengh: (integer); length to nearest foot from end to end of tapers.

Rtrnawidth: (integer); width to nearest foot from shoulder to outside edge.

Rtrnasurf: Paved asphalt, Processed gravel, Bank run gravel, Natural surface, Other; the primary surface material of the turn around.

Water Hole

Rwholidnum: (text); draft’s ID number if known, otherwise blank.

Rwholesize: (text); estimated size in feet (Length x Width x Depth).

Rwholacces: Good, Fair, Poor; ability of a fire vehicle to be able to draft water.  Good = no difficulty drafting; Fair = could draft with difficulty; Poor = not accessible for a vehicle to draft water.

Rwholsuveg: Good, Fair, Poor; surrounding vegetation.  Good = vegetation cleared; Fair = vegetation not bad but could use clearing; Poor = failed, vegetation needs clearing.

Rwholsedim: Good, Fair, Poor; sedimentation in water hole.  Good = free of sediment, clean bottom; Fair = half full of sediment, needs cleaning; Poor = full of sediment, needs cleaning.

Trail Sign(s)/Kiosk – [this is a sign, a group of signs, or a kiosk, NOT including intersection number signs, trail markers, boundary markers, road signs, or trail/road name signs; these are all taken care of by other feature types or their attributes]

Tsigncond: Good, Fair, Poor; the physical condition (and legibility) of the sign/kiosk.

Tsigninfo: (True/False); whether there is an informational sign present.

Tsignintrp: (True/False); whether there is an interpretive sign present.

Tsignregul: (True/False); whether there is a regulatory sign present.

Tsignkiosk: (True/False); whether there is a kiosk structure present.

Trail Structure/Erosion Control – [a manmade structure associated with the trail that was constructed for the purpose of erosion control or to maintain the physical integrity of the trail, etc.]

Tstrustype: Steps, Cribbing, Retaining wall, Guardrail, Drainage dip, Waterbar, Checkdam, Turnpike, Corduroy, Switchback/climbing turn, Other; type of trail structure or erosion control.

Tstrumater: Rock, Mineral soil, “Wood, untreated”, “Wood, treated”, Metal, Plastic, Other; the primary material of which the structure is made.

Tstruscond: Good, Fair, Poor; the physical condition of the structure.

Tstrusfunc: Good, Fair, Poor; the functional condition of the structure—how well the structure is performing its job.

Trail Damage – [if damage is longer than 50 feet, there should also be a separate line segment (with attributes showing condition of trail) to show the extent of the damage (the trail line leading up to the damage should stop at the beginning of the damage, then a new line for the duration of the damage, then a new line starting after the damage).  Regardless of the length of damage, there should be a Trail Damage point – so for a long damaged area, you will have both a line and a point]

Tdamgtype: Washout, Water/mud hole, Large boulder/ledge, Ruts, Protruding rocks, Protruding roots, Braided trail, Other; type of trail damage.

Tdamglengt: (integer); length of the damaged area in feet.

Tdamgdepth: None, Loss of surface vegetation, Exposed roots, < 2", 2" - 8", 9" - 16", 17" - 32", 33" - 60", > 60"; depth of the damaged area (if applicable).

Trail Stream Crossing – [anywhere a trail crosses a water body]

Tstrxctype: Bridge, Boardwalk, Culvert, Unimproved ford, Improved ford, Bog bridge, Other; the type of stream crossing.

Tstrxmater: Rock, “Wood, untreated”, “Wood, treated”, Metal, Plastic, Other; the primary material of which the stream crossing is made.

Tstrxwtype: Stream, Wetland, Spring, Seasonal Runoff, Other; the type of water body the trail is crossing.

Tstrxlengt: (integer); the length, to the nearest foot, of the crossing.

Tstrxscond: Good, Fair, Poor; the physical condition of the structure.

Tstrxccond: Good, Fair, Poor; the functional condition of the crossing (how easy is it to cross?).

Trail Meets Utility Right-Of-Way

Tutixutype: Gas, Electric, Phone, Other; type of utility line.

Tutixconfg: Above ground, Below ground; configuration of the utility line.

Tutiximpct: (True/False); does the trail crossing negatively impact the utility line?

Trail Crosses Stone Wall

No additional attributes necessary.

Trail Crosses DCR Boundary

Tbndxspdcr: (True/False); is there a DCR property sign at this boundary crossing?

Tbndxsppri: (True/False); is there a private-property sign at this boundary crossing?

Building

No additional attributes necessary.

Shelter

No additional attributes necessary.

Campsite

No additional attributes necessary.

Picnic Table/Area

Picninum: (integer); the number of picnic tables in this picnic area.

Picnimater: Wood, Plastic, Metal, Stone, Concrete, Other; the primary material of which the picnic table is made.

Bench(es)

Benchnum: (integer); the number of benches in this immediate area (usually 1, but can be more).

Benchmater: Wood, Plastic, Metal, Stone, Concrete, Other; the primary material of which the bench is made.

Vista

No additional attributes necessary.

Recreational Site

No additional attributes necessary.

Historic/Cultural Site

No additional attributes necessary.

Noteworthy Natural Feature

No additional attributes necessary.

Encroachment Issue

Encrotype: “Trash, small”, “Trash, large”, Cut vegetation, Vehicle/boat, Structure, Motorized trail, Other trail, Other; the type of encroachment.  It is recommended that you type a comment into the Comment field to describe the encroachment further.

Boundary Corner

Boundtype: Stone monument, Concrete monument, Town/state line monument, Iron pipe, Iron rod/rebar/pin, Drill hole, Stake and stones, Pile of stones, Survey monument, Land court disk, Stone wall intersection-no marker, Stone wall corner-no marker, Fence corner-no marker, Unmarked point along boundary, Boundary tree, Other; the type of monumentation found at this boundary corner.

Other

No additional attributes necessary; Comments required.

Fields that are available for all Types:

Comments: (text); the user can optionally enter a comment if there is anything unusual about the point or anything that needs to be explained more clearly than is possible with the standard attributes.

Source: (text); this is entered back in the office, not in the field.  It indicates the source of the data, including whether the point was GPSed or digitized, and whether it was differentially corrected or not.

GPS_Date: (text); holds the date the feature was GPSed in the field.  Autogenerated by the GPS unit.

GPS_Time: (text); holds the time the feature was GPSed in the field.  Autogenerated by the GPS unit.

Phototaken: (True/False); indicates whether the user took a photo at this point.  Used rarely.

Photo_id: (text); if the user took a photo, they can type the number of the photo here for later correlation.  Used rarely.

Field_note: (integer); refers to a numbered note the user can write on paper (on a clipboard or notebook) if they have comments that won’t fit in the Comments field.  Fill this field with a number and then write that same number on your clipboard followed by the field note.  Used rarely.

Additional attributes, including ones that indicate lines and points that should be deleted, trail names, and allowed trail uses, will be added to the files in the office.